FAMEPIX: The Pioneering French High-Resolution Hyperspectral 3D X-Ray Microscope

Julio Cesar da Silva1,2,3 & FAMEPIX collaboration

1Univ. Grenoble Alpes, CNRS, Grenoble INP, Institut Néel, 38000 Grenoble, France

2French CRG beamline FAMEPIX, ESRF, 38000 Grenoble, France

3Unité GDR2116 Coherex, 75060 Paris, France

Courriel : julio-cesar.da-silva@neel.cnrs.fr

While data acquisition in Ptychographic X-ray Computed Tomography (PXCT) has advanced, achieving hyperspectral nanoimaging still necessitates further speed enhancements. Identifying element locations and chemical speciation within a sample’s microstructure demands numerous PXCT experiments at various photon energies. To enhance this capability, we can optimize instrumentation for expedited data collection and adjust analysis methods to minimize data requirements without compromising image quality. This can be achieved through instrumentation upgrades and refined data processing techniques that optimize data volume while preserving high resolution. We have already made progress in data analysis methods [1,2,3]. Presently, the MAGNIFIX project funding enables the construction of the new endstation FAMEPIX at the ESRF. FAMEPIX integrates all aspects of spectral ptychography and spectral PXCT, making it the pioneer X-ray microscope in France with these capabilities. Advancements in instrumentation broaden the applications, while the need to characterize diverse samples also dictates the requirements for instrumentation. Construction of the beamline has commenced, and the experimental nanoprobe is being prepared in the manufacturer’s workshop. FAMEPIX will supersede the preceding FAME beamline at the ESRF port BM30. FAMEPIX will benefit from the small stable source of this port of the upgraded ESRF-EBS, which is approximately 40µm × 40µm. In this presentation, I will introduce FAMEPIX, the novel F-CRG beamline that will confer PXCT with spectral capabilities, thereby unlocking novel opportunities for characterizing heterogeneous materials.

REFERENCES:

1) J. da Silva et al., Opt. Express, 2015, 23, 33812-33821.

2) A. Kulow et al., J. Synchrotron Rad., 2024, 31(4), 867-876.

3) R. Boudjehem et al., J. Synchrotron Rad., 2024, 31(2), 399-408.